6) How many bits are required to address a 1M × 8 main memory if

- a)  Main memory is byte-addressable?

- b)  Main memory is word-addressable?

9. How many 256x8 RAM chips are needed to provide a memory capacity of 4096 bytes?

a) How many bits will each address contain?

b) How many lines must go to each chip?

c) How many lines must be decoded for the chip select inputs? Specify the size of the decoder.

10. Suppose that a 2M x 16 main memory is built using 256K × 8 RAM chips and memory is word-addressable.

- a)  How many RAM chips are necessary?

- b)  If we were accessing one full word, how many chips would be involved?

- c)  How many address bits are needed for each RAM chip?

- d)  How many banks will this memory have?

- e)  How many address bits are needed for all of memory?

- f)  If high-order interleaving is used, where would address 14 (which is E in hex) be

located?

- g)  Repeat Exercise 9f for low-order interleaving.

10. Suppose that a 2M x 16 main memory is built using 256K × 8 RAM chips and memory is word-addressable.

- a)  How many RAM chips are necessary?

- b)  If we were accessing one full word, how many chips would be involved?

- c)  How many address bits are needed for each RAM chip?

- d)  How many banks will this memory have?

- e)  How many address bits are needed for all of memory?

- f)  If high-order interleaving is used, where would address 14 (which is E in hex) be

located?

- g)  Repeat Exercise 9f for low-order interleaving.

11. Suppose that a 16M x 16 main memory is built using 512K × 8 RAM chips and memory is word-addressable.

- a)  How many RAM chips are necessary?

- b)  If we were accessing one full word, how many chips would be involved?

- c)  How many address bits are needed for each RAM chip?

- d)  How many banks will this memory have?

- e)  How many address bits are needed for all of memory?

- f)  If high-order interleaving is used, where would address 14 (which is E in hex) be

located?

- g)  Repeat Exercise 9f for low-order interleaving.

13. A digital computer has a memory unit with 24 bits per word. The instruction set consists of 150 different operations. All instructions have an operation code part (opcode) and an address part (allowing for only one address). Each instruction is stored in one word of memory.

- a)  How many bits are needed for the opcode?

- b)  How many bits are left for the address part of the instruction?

- c)  What is the maximum allowable size for memory?

- d)  What is the largest unsigned binary number that can be accommodated in one word of

memory?